5. Final Optimization Findings

5.1 Comparison with the Original DWSIM Dataset

The best-performing condition among the original 73 DWSIM simulations was:

| Variable | Best Original DWSIM Condition |
| --- | --- |
| Boilup | 0.47 |
| Bottom Flow C2 | 0.09 kg/s |
| RR2 | 1.0 |
| Pressure C2 | 1.0 |
| Temp C1 | 256°C |
| Ethanol Purity | 96.15 wt% |
| Ethanol Recovery | 98.69% |
| EG Recovery | 99.88% |
| Total Energy | 8611.7 kW |
| PPI | 0.95127 |

This condition represents the highest PPI obtained directly from the original 73 DWSIM simulations.

5.2 Comparison with AI-Recommended Conditions

The AI optimization produced a different operating region. Its five highest-ranked candidates were concentrated around:

- Boilup ≈ 0.58

- Bottom Flow C2 = 0.09 kg/s

- RR2 = 2.5

- Pressure C2 = 1.4 bar

- Temp C1 ≈ 237–240°C

After independent DWSIM validation, the best of these five candidates achieved:

PPI = 0.90534

The comparison is:

| Performance | Original Best DWSIM | Best AI Candidate After DWSIM |
| --- | --- | --- |
| Boilup | 0.47 | 0.579 |
| Bottom Flow C2 | 0.09 | 0.09 |
| RR2 | 1.0 | 2.5 |
| Pressure C2 | 1.0 | 1.4 |
| Temp C1 | 256°C | 239°C |
| DWSIM PPI | 0.95127 | 0.90534 |

The original DWSIM condition therefore remains the highest-PPI condition in the available simulation dataset.

5.3 Why Did the AI Find a Different Region?

The difference between the original optimum and the AI-recommended region provides an important insight into the limitations of the surrogate model.

The original best condition operates at:

Temp C1 = 256°C

whereas the engineering-guided AI search deliberately restricted temperature to:

180–240°C

because the engineering sensitivity study identified a severe deterioration in ethanol purity near the upper temperature region.

The original optimum also used:

RR2 = 1.0

whereas the engineering-guided AI optimization fixed:

RR2 = 2.5

Therefore, the AI optimization was not searching the same design space as the original global 73-run dataset.

Consequently, the AI result and the original DWSIM optimum should not be interpreted as a direct head-to-head optimization comparison.

5.4 Important Engineering Interpretation

The original DWSIM dataset indicates that the highest PPI occurs at a condition combining:

- moderate boilup,

- high solvent recovery,

- low Column 2 reflux ratio,

- relatively high Column 1 temperature,

- and low total energy consumption.

The AI optimization, on the other hand, identified a high-performing region within the more conservative engineering constraints.

This demonstrates an important distinction between:

Dataset optimum

and

Engineering-constrained AI optimum.

The dataset optimum maximizes the defined PPI among the simulations already performed.

The AI optimization searches for promising conditions within a deliberately restricted operating region based on the engineering interpretation of the process.

5.5 Validation of the AI Screening Approach

Although the AI model did not reproduce the absolute best PPI from the original dataset, it successfully identified a region containing multiple high-performing operating conditions.

All five AI-selected candidates converged in DWSIM and produced PPI values above approximately 0.90.

The validated values were:

| AI Rank | Predicted PPI | DWSIM PPI |
| --- | --- | --- |
| 1 | 0.93637 | 0.90022 |
| 2 | 0.93305 | 0.90410 |
| 3 | 0.93172 | 0.90289 |
| 4 | 0.93139 | 0.90394 |
| 5 | 0.93114 | 0.90534 |

This indicates that the surrogate model was useful for identifying a promising operating region, even though it systematically overpredicted the absolute PPI of the selected candidates.

5.6 Final Validated AI Condition

Among the five AI recommendations, the highest DWSIM-validated PPI was obtained for:

| Parameter | Value |
| --- | --- |
| Boilup | 0.579 |
| Bottom Flow C2 | 0.090 kg/s |
| RR2 | 2.5 |
| Pressure C2 | 1.4 bar |
| Temp C1 | 239°C |
| DWSIM PPI | 0.90534 |

This condition can therefore be reported as the:

Best DWSIM-validated condition obtained from the AI-guided search.

It should not be described as the global optimum of the complete process because the original 73-run dataset contains a higher-PPI condition.

5.7 Final Project Finding

The combined DWSIM, engineering-analysis and machine-learning workflow produced two important outcomes.

Best condition within the original DWSIM dataset

PPI = 0.95127

at:

Boilup = 0.47, Bottom Flow C2 = 0.09 kg/s, RR2 = 1.0, Pressure C2 = 1.0 bar, Temp C1 = 256°C.

Best condition obtained from the engineering-guided AI search

DWSIM-validated PPI = 0.90534

at:

Boilup = 0.579, Bottom Flow C2 = 0.09 kg/s, RR2 = 2.5, Pressure C2 = 1.4 bar, Temp C1 = 239°C.

The first represents the best observed simulation result, while the second demonstrates the ability of the ML-based workflow to rapidly identify and validate promising operating conditions within a constrained design space.

5.8 Overall Project Conclusion

The project demonstrates a hybrid chemical-engineering and machine-learning workflow for the analysis and screening of an ethanol–water extractive distillation process using ethylene glycol as an entrainer.

The 73 DWSIM simulations were first used to understand the process behaviour and identify important operating variables. Engineering analysis showed that Column 1 temperature and Column 2 bottom flow were the most influential variables within the investigated dataset.

A PPI was then developed to combine ethanol purity, combined recovery performance and energy consumption into a single performance measure.

Machine learning was subsequently used to construct a surrogate model for rapid PPI screening. Linear Regression achieved the highest mean cross-validation R² among the tested models, approximately 0.72.

The trained model was then used to screen approximately 100,000 feasible operating conditions. The highest-ranked candidates were independently simulated in DWSIM.

The AI recommendations consistently converged and produced validated PPI values above 0.90, demonstrating that the surrogate model could be used as an efficient screening layer around the rigorous process simulator.

However, the original 73-run DWSIM dataset contained a higher PPI of 0.95127. Therefore, the main contribution of the AI stage is not that it discovered a better optimum than the original simulation study, but that it demonstrated a workflow for rapid exploration and screening of a large operating space followed by rigorous process-level validation.

The overall methodology can therefore be summarized as:

DWSIM → Engineering Analysis → PPI → Machine Learning → Large-Scale Screening → DWSIM Validation

This hybrid approach combines physical process modelling with data-driven screening while retaining DWSIM as the final source of process validation.