6. Overall Engineering Findings and Conclusions

6.1 Overall Process Findings

The combined DWSIM simulation, engineering analysis, and machine-learning study showed that the performance of the extractive-distillation system is controlled by a trade-off between ethanol purity, ethanol recovery, ethylene-glycol recovery, and energy consumption.

The PPI used in this study was defined as:

- 40% ethanol purity

- 40% average recovery, where average recovery is the mean of ethanol and EG recovery

- 20% energy performance

This formulation allowed the process to be evaluated using a single performance measure while still considering the major engineering objectives.

The analysis of 73 converged operating cases showed that the process does not have a simple "higher operating variable = better performance" relationship. Different variables influence different aspects of the separation, making multi-objective evaluation necessary.

6.2 Most Influential Operating Variables

The machine-learning feature-importance analysis identified the following ranking:

| Operating Variable | Feature Importance |
| --- | --- |
| Temp C1 | 0.5061 |
| Bottom Flow C2 | 0.3642 |
| Boilup | 0.0809 |
| RR2 | 0.0262 |
| Pressure C2 | 0.0226 |

Column 1 feed temperature was the most influential variable, followed by Column 2 bottom flow.

This was consistent with the engineering correlation analysis. In particular:

- Temp C1 vs Ethanol Recovery: r = 0.837

- Bottom Flow C2 vs EG Recovery: r = 0.994

- Bottom Flow C2 vs PPI: r = 0.641

- Temp C1 vs PPI: r = 0.562

- Total Energy vs PPI: r = −0.699

Therefore, the ML analysis did not simply provide a numerical prediction; it helped identify the variables that deserve the greatest engineering attention.

6.3 Column 1 Feed Temperature

Feed temperature was found to have the strongest influence on the process.

Between approximately 180°C and 240°C, ethanol purity remained within a relatively stable high-performance region. However, increasing the temperature toward 250°C resulted in a severe reduction in ethanol purity, reaching approximately 68.8 wt%.

At the same time, ethanol recovery increased substantially with temperature, reaching approximately 99% near the upper end of the investigated range.

This demonstrates a clear purity–recovery trade-off.

Increasing temperature can improve ethanol recovery, but excessive thermal input can compromise the separation quality. Therefore, temperature should not be maximized simply to obtain higher recovery.

Engineering implication: Column 1 feed temperature should be controlled within a stable operating region rather than operated near the observed high-temperature failure region.

6.4 Column 2 Bottom Flow and Solvent Recovery

Column 2 bottom flow showed the strongest relationship with ethylene-glycol recovery.

The correlation between Bottom Flow C2 and EG Recovery was:

r = 0.994

Increasing the bottom flow toward approximately 0.09 kg/s resulted in EG recovery approaching 100%.

This is particularly important because ethylene glycol acts as the entrainer in the extractive-distillation process. Effective recovery therefore helps maintain the solvent loop and reduces solvent loss.

The variable was also the second-most important feature in the ML analysis.

Engineering implication: Maintaining an appropriate Column 2 bottom flow is important not only for the PPI but also for maintaining effective solvent recovery.

6.5 Column 1 Boilup

The boilup study demonstrated that increasing boilup does not continuously improve product quality.

A boilup ratio around 0.44 produced the maximum observed ethanol purity of approximately 97.68 wt%.

Beyond this region, increasing boilup resulted in decreasing ethanol purity, despite improvements in ethanol recovery.

Therefore, excessive vapor generation does not necessarily improve the overall separation.

Engineering implication: The boilup ratio should be selected based on the required balance between purity, recovery, and energy rather than maximizing boilup.

6.6 Column 2 Reflux Ratio and Pressure

RR2 and Column 2 pressure had comparatively low feature importance in the developed ML model:

- RR2: 0.0262

- Pressure C2: 0.0226

This does not mean that these variables are physically unimportant in all operating situations. Rather, within the specific range and dataset investigated, their influence on PPI was smaller than that of temperature and bottom flow.

The best observed PPI case occurred at:

- RR2 = 1.0

- Pressure C2 = 1.0 bar

with a PPI of:

0.9513

This result suggests that unnecessarily increasing reflux or pressure does not automatically produce better overall process performance.

6.7 Best Observed DWSIM Operating Condition

Among the 73 investigated operating conditions, the highest observed PPI was:

PPI = 0.9513

at:

| Parameter | Value |
| --- | --- |
| Boilup | 0.47 |
| Bottom Flow C2 | 0.09 kg/s |
| RR2 | 1.0 |
| Pressure C2 | 1.0 bar |
| Temp C1 | 256°C |
| Ethanol Purity | 96.15 wt% |
| Ethanol Recovery | 98.69% |
| EG Recovery | 99.88% |
| Total Energy | 8611.7 kW |
| PPI | 0.9513 |

This should be described as the best observed operating condition within the investigated simulation dataset, rather than claiming it as the universal/global optimum.

6.8 Machine-Learning Findings

Four regression algorithms were evaluated using cross-validation.

| Model | Mean R² | Standard Deviation |
| --- | --- | --- |
| Linear Regression | 0.7212 | 0.1680 |
| Extra Trees | 0.6625 | 0.1991 |
| Gradient Boosting | 0.6590 | 0.2099 |
| Random Forest | 0.6263 | 0.2088 |

Linear Regression produced the highest mean cross-validation R² among the tested models.

However, the relatively large variation between validation folds indicates that the available dataset of 73 simulations is limited. Consequently, the trained model is best treated as a surrogate model for screening and exploration, rather than a replacement for DWSIM.

This is also consistent with the broader use of surrogate modelling in chemical-process optimization, where inexpensive predictive models are used to explore operating spaces before selected candidates are evaluated using higher-fidelity simulations.

6.9 AI-Assisted Optimization

The engineering-guided AI optimization approach generated 100,000 candidate operating conditions within the defined engineering limits.

The optimization focused on the more influential variables while keeping:

- RR2 = 2.5

- Pressure C2 = 1.4 bar

fixed.

The AI identified a high-performing region approximately around:

- Boilup = 0.578–0.588

- Bottom Flow C2 = 0.09 kg/s

- Temp C1 = 237–240°C

The five highest-ranked AI candidates were then validated independently in DWSIM.

The DWSIM PPI values were:

| Candidate | AI PPI | DWSIM PPI |
| --- | --- | --- |
| 1 | 0.9364 | 0.9002 |
| 2 | 0.9330 | 0.9041 |
| 3 | 0.9317 | 0.9029 |
| 4 | 0.9314 | 0.9039 |
| 5 | 0.9311 | 0.9053 |

The AI predictions were higher than the DWSIM results by approximately 0.026–0.036 PPI units.

This difference demonstrates an important limitation of the surrogate model: AI predictions should not be treated as final physical results without simulator validation.

At the same time, all five candidates remained high-performing in DWSIM, demonstrating that the AI model was able to identify a promising operating region.

Surrogate-based optimization is particularly useful for reducing the number of expensive high-fidelity evaluations, but validation against the underlying simulator remains an important part of the workflow.

6.10 Key Engineering Insight

The most important finding from the complete study is that the best operating strategy cannot be determined by optimizing a single process variable.

For example:

- Higher temperature can improve ethanol recovery but eventually destroys purity.

- Higher boilup can improve recovery but can reduce ethanol purity.

- Higher Column 2 bottom flow improves EG recovery.

- Higher energy consumption generally reduces PPI.

- The best PPI therefore comes from balancing several competing objectives.

The use of the PPI allowed these competing effects to be considered simultaneously.

6.11 Final Conclusion

This project developed a hybrid process simulation–engineering analysis–machine learning–AI optimization workflow for ethanol–water extractive distillation using ethylene glycol as the entrainer.

The 73 DWSIM simulations provided the process dataset. Engineering analysis was then used to identify operating trends and important process variables. Machine-learning models were subsequently evaluated as surrogate models for PPI prediction, and the selected model was used to screen 100,000 feasible operating conditions.

The analysis identified Column 1 temperature and Column 2 bottom flow as the two most influential variables, while the results also revealed important purity–recovery and energy–performance trade-offs.

The highest observed PPI within the original DWSIM dataset was 0.9513, obtained at a boilup ratio of 0.47, Column 2 bottom flow of 0.09 kg/s, RR2 of 1.0, Column 2 pressure of 1.0 bar, and Column 1 temperature of 256°C.

The AI optimization identified a different high-performing operating region. Although its PPI predictions were somewhat optimistic, independent DWSIM validation of the top five candidates produced PPI values between 0.9002 and 0.9053.

Therefore, the final outcome of the project is not that the AI replaced process simulation, but that AI can be used as a rapid screening layer around a process simulator:

DWSIM → Engineering Analysis → ML Surrogate → AI Screening → DWSIM Validation

This workflow provides a practical framework for exploring a large operating space while retaining the physical process simulator as the final validation tool. Such simulator–surrogate workflows are an established direction for reducing the computational burden of process optimization.

Final project statement

The study demonstrates how chemical-engineering knowledge and data-driven methods can be combined to analyse and optimize an extractive-distillation process. DWSIM provides the physical simulation foundation, engineering analysis identifies meaningful operating behaviour, machine learning provides rapid prediction, and DWSIM validation ensures that AI-selected conditions remain physically meaningful.