4. AI Optimization Results

4.1 Objective

The final stage of the project used the trained machine-learning model as a computationally inexpensive screening tool for identifying promising operating conditions.

Rather than treating the ML prediction as the final process result, the optimization followed an engineering-guided surrogate approach:

Engineering constraints → Candidate generation → ML screening → Top candidates → DWSIM validation

This approach allowed a large number of feasible operating conditions to be evaluated computationally before returning the most promising conditions to the rigorous DWSIM process model.

4.2 Engineering-Guided Search Strategy

The second optimization approach was selected as the primary AI optimization method.

Based on the engineering analysis, the search was restricted to the validated operating region:

| Variable | Search Range |
| --- | --- |
| Boilup | 0.35–0.59 |
| Bottom Flow C2 | 0.06–0.09 |
| RR2 | 2.5 (fixed) |
| Pressure C2 | 1.4 bar (fixed) |
| Temp C1 | 180–240°C |

The search was intentionally not allowed to explore unrestricted operating conditions.

The sampling was also biased toward regions identified as promising from the previous engineering analysis. In particular, higher boilup, higher bottom flow, and the upper part of the safe temperature region were sampled more densely.

This created an engineering-guided optimization strategy rather than a purely random search.

4.3 Large-Scale AI Screening

A total of approximately 100,000 candidate operating conditions were generated within the defined engineering limits.

The trained ML model was then used to predict the PPI for each feasible candidate.

The candidates were ranked according to predicted PPI, and the highest-ranked conditions were retained for further evaluation.

This provides a major computational advantage: the surrogate model can evaluate a very large number of candidate conditions much faster than repeatedly solving the complete process simulation.

Surrogate-based process optimization commonly follows this general principle: simulation data are used to build a computationally cheaper approximation, which is then used to search the design space before promising solutions are verified using the original simulator.

4.4 Top AI-Recommended Operating Conditions

The five highest-ranked conditions obtained from the engineering-guided AI search were:

| Rank | Boilup | Bottom Flow C2 | RR2 | Pressure C2 | Temp C1 | Predicted PPI |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 0.588 | 0.090 | 2.5 | 1.4 | 239°C | 0.9364 |
| 2 | 0.578 | 0.090 | 2.5 | 1.4 | 240°C | 0.9330 |
| 3 | 0.580 | 0.090 | 2.5 | 1.4 | 239°C | 0.9317 |
| 4 | 0.588 | 0.090 | 2.5 | 1.4 | 237°C | 0.9314 |
| 5 | 0.579 | 0.090 | 2.5 | 1.4 | 239°C | 0.9311 |

A clear pattern can be observed in the AI recommendations.

All five conditions selected:

- Bottom Flow C2 = 0.09 kg/s

- RR2 = 2.5

- Pressure C2 = 1.4 bar

- Boilup ≈ 0.58

- Temp C1 ≈ 237–240°C

This is particularly interesting because the AI did not produce five completely different operating points. Instead, it identified a narrow high-performance operating region.

4.5 Independent DWSIM Validation

The five AI-recommended conditions were subsequently simulated independently in DWSIM.

All five simulations converged successfully.

The resulting DWSIM performance indices were:

| Rank | AI Predicted PPI | DWSIM PPI | Absolute Difference |
| --- | --- | --- | --- |
| 1 | 0.9364 | 0.9002 | 0.0362 |
| 2 | 0.9330 | 0.9041 | 0.0290 |
| 3 | 0.9317 | 0.9029 | 0.0288 |
| 4 | 0.9314 | 0.9039 | 0.0274 |
| 5 | 0.9311 | 0.9053 | 0.0258 |

The average absolute difference between the AI predictions and DWSIM results was approximately:

0.0294 PPI points

The average absolute percentage difference was approximately:

3.26%

The AI predictions were consistently higher than the corresponding DWSIM values.

Therefore, the ML model showed a tendency to overpredict the absolute PPI in this high-performance region.

4.6 What the Validation Actually Demonstrates

The most important conclusion is not that the AI model perfectly predicted the PPI.

It did not.

Instead, the validation demonstrates that the AI model was able to identify a physically feasible and high-performing operating region that remained viable when evaluated using the underlying DWSIM model.

This distinction is important because the ML model was trained using only 73 simulations and achieved a mean cross-validation R² of approximately 0.72.

The final DWSIM validation therefore acts as a safeguard against relying exclusively on surrogate predictions.

4.7 Engineering Pattern Revealed by the AI Search

The AI optimization produced a useful engineering observation.

The recommended conditions consistently moved toward:

High Bottom Flow C2

and

High but controlled Boilup

while maintaining:

Temp C1 near the upper end of the validated safe region.

This agrees with the earlier engineering analysis:

- Higher Bottom Flow C2 improves EG recovery.

- Higher boilup can improve recovery but eventually compromises ethanol purity.

- Increasing temperature improves ethanol recovery but becomes dangerous near the high-temperature limit.

- Therefore, the optimization naturally moves toward the boundary of the useful operating region without crossing the known temperature limit.

The AI search therefore did not simply produce an arbitrary mathematical optimum. Its recommendations were broadly consistent with the process trends already identified from the DWSIM sensitivity analysis.

4.8 Important Limitation of the AI Optimization

The highest AI-predicted PPI of 0.9364 should not be reported as the experimentally or simulation-verified optimum.

The corresponding DWSIM value was 0.9002.

Similarly, the other four predictions were also lower when checked using DWSIM.

Therefore, the AI results should be described as:

AI-recommended operating conditions

rather than:

globally optimal operating conditions

The optimization is limited by:

- The relatively small training dataset of 73 simulations.

- The predictive uncertainty of the surrogate model.

- The fact that the search space is restricted to the validated operating region.

- The need for DWSIM verification of promising candidates.

4.9 Final AI Optimization Finding

The engineering-guided ML optimization successfully reduced a large feasible operating space to a small set of promising operating conditions.

The top five AI recommendations all converged when independently evaluated in DWSIM, with validated PPI values between approximately 0.900 and 0.905.

The best DWSIM-validated PPI among the five candidates was:

PPI = 0.9053

corresponding to:

- Boilup = 0.579

- Bottom Flow C2 = 0.090 kg/s

- RR2 = 2.5

- Pressure C2 = 1.4 bar

- Temp C1 = 239°C

The AI therefore demonstrated its usefulness as a rapid operating-region screening tool, while DWSIM provided the final process-level verification.

4.10 Overall Conclusion of the AI Stage

The final optimization demonstrates a practical hybrid workflow:

DWSIM provides physical process behaviour.

Engineering analysis identifies meaningful operating regions.

Machine learning rapidly screens a large number of candidate conditions.

DWSIM validates the most promising AI recommendations.

This approach avoids treating the ML model as a replacement for process simulation. Instead, machine learning acts as an acceleration layer around the rigorous process model.

The final validated result therefore comes from the combination of engineering knowledge, surrogate modelling, and DWSIM simulation, rather than from the AI prediction alone.