3. Machine Learning Results

3.1 Machine Learning Objective

The objective of the machine-learning stage was to develop a surrogate model capable of predicting the Process Performance Index (PPI) from the five controllable operating variables:

- Boilup Ratio

- Bottom Flow C2

- RR2

- Pressure C2

- Temp C1

The purpose of the model was not to replace DWSIM, but to provide a fast method for screening a large number of feasible operating conditions before performing detailed DWSIM simulations.

3.2 Dataset Used for Machine Learning

The prepared dataset contained 73 converged DWSIM simulations.

The five operating variables were used as model inputs, while the calculated PPI was used as the target variable.

The dataset was divided using an 80:20 training/testing split during the model-development stage.

Because the dataset was relatively small, model selection was additionally performed using cross-validation. Cross-validation evaluates the model over multiple training/validation splits rather than relying on a single split, giving a more robust estimate of model performance.

3.3 Model Comparison

Four regression algorithms were evaluated:

| Model | Mean R² | Standard Deviation |
| --- | --- | --- |
| Linear Regression | 0.7212 | 0.1680 |
| Extra Trees | 0.6625 | 0.1991 |
| Gradient Boosting | 0.6590 | 0.2099 |
| Random Forest | 0.6263 | 0.2088 |

Linear Regression produced the highest mean cross-validation R² of approximately 0.72 and was therefore selected as the primary surrogate model.

The R² score represents the proportion of target variance explained by the model relative to a constant baseline. An R² of 1 represents a perfect prediction, while 0 corresponds to a constant mean-prediction baseline.

3.4 Model Selection Finding

The most important result from model comparison was that the more complex tree-based models did not outperform the simpler Linear Regression model on this dataset.

This suggests that, over the investigated operating range, a substantial portion of the variation in PPI could be represented using relatively simple relationships between the selected operating variables and PPI.

Therefore, increasing model complexity did not automatically produce better predictive performance.

The Linear Regression model was consequently retained for the AI optimization stage.

3.5 Feature Importance Analysis

The feature-importance analysis identified the following relative contributions:

| Operating Variable | Importance |
| --- | --- |
| Temp C1 | 0.5061 |
| Bottom Flow C2 | 0.3642 |
| Boilup | 0.0809 |
| RR2 | 0.0262 |
| Pressure C2 | 0.0226 |

The analysis identified Column 1 feed temperature as the dominant variable, followed by Column 2 bottom flow.

Together, these two variables accounted for approximately 87% of the reported feature importance.

This result agrees strongly with the independent engineering analysis:

- Temp C1 showed a correlation of 0.97 with ethanol recovery.

- Bottom Flow C2 showed a correlation of 0.99 with EG recovery.

The agreement between the engineering analysis and the ML feature-importance analysis provides additional confidence that these two variables are important within the simulated operating space.

3.6 Relationship Between Engineering Analysis and Machine Learning

An important outcome of the project was that the ML analysis did not operate independently of the chemical-engineering analysis.

The engineering analysis identified the major process relationships first. The ML model then provided a quantitative way of representing these relationships for rapid prediction.

For example:

Engineering analysis

Temp C1 strongly influences ethanol recovery.

ML analysis

Temp C1 has the highest reported feature importance at approximately 0.51.

Similarly:

Engineering analysis

Bottom Flow C2 has a very strong relationship with EG recovery.

ML analysis

Bottom Flow C2 has the second-highest reported feature importance at approximately 0.36.

This agreement was used to guide the subsequent AI optimization strategy.

3.7 Model Limitations

The machine-learning results should be interpreted within the limitations of the available dataset.

Only 73 DWSIM simulations were available for model development. Although cross-validation was used for model selection, the relatively large standard deviations of the cross-validation scores indicate that model performance varies across different subsets of the data.

For example, Linear Regression achieved:

Mean R² = 0.7212

with:

Standard deviation = 0.1680

Therefore, the model should not be interpreted as an exact replacement for the underlying DWSIM simulation.

Another limitation is that the optimization search is restricted to the operating ranges represented by the simulation dataset. Predictions outside these ranges would constitute extrapolation and were therefore avoided.

3.8 Role of the ML Model in the Final Workflow

The ML model was ultimately used as a surrogate screening model rather than as the final authority on process performance.

The final workflow was:

DWSIM simulations

↓

Engineering analysis

↓

PPI development

↓

ML model training and comparison

↓

Engineering-guided generation of feasible operating conditions

↓

100,000 AI-screened candidates

↓

Top AI recommendations

↓

Independent DWSIM validation

This approach combines the speed of machine learning with the physical-process validation provided by DWSIM.

3.9 Overall Machine Learning Conclusion

The machine-learning stage demonstrated that the five selected operating variables contain sufficient information to develop a useful surrogate model for PPI screening.

Linear Regression achieved the highest mean cross-validation performance among the tested models, with an R² of approximately 0.72.

The ML feature-importance analysis identified Temp C1 and Bottom Flow C2 as the dominant variables, consistent with the independent engineering analysis.

However, because the dataset contained only 73 simulations and the cross-validation score showed noticeable variation, the ML model was treated as a candidate-generation and screening tool rather than a replacement for rigorous process simulation.

This limitation directly motivated the final validation step: the highest-ranked AI operating conditions were returned to DWSIM and independently simulated before being considered promising process conditions.